Functions:

- OSS_samplecode

- align2

Function call:

[Mask Rr Rf Rec] = OSS_samplecode(diffraction_pattern,[support_size1 support_size2],iterations,beta,showimage,model);

Inputs:

diffraction_pattern: Diffraction pattern of the oversampled vesicle image, with the central speckle removed, and 5% Poisson noise (Rnoise) added as defined above.

support_size1 [52]: Size of rectangular support along the first (y) dimension.

support_size2 [28]: Size of rectangular support along the second (x) dimension.

iterations [2000]: Total number of iterations performed by the OSS reconstruction.

beta [0.9]: Parameter used in a typical HIO reconstruction (see Fienup, Opt. Let. 1978).

showimage [1]: Defines whether or not images showing progress are displayed (1 is shown, 0 not shown).

model: Image of the oversampled vesicle model used for the OSS reconstruction.

Outputs:

Mask: An image of the mask used as the support in the OSS reconstruction.

Rf: Error measurement in Fourier space, defined above, computed every 2 iterations.

Rr: Error measurement in real space, defined above, computed alongside Rf.

Rec: Final result image obtained from the OSS reconstruction.